Made by Piter Garcia

Farmingdale University of New York

Subject:

EET 111

Prof.:

Adam A. Fillios

Date:

November, 05, 2008

Piter Garcia

Manual Lab Experiment #5

Mesh Analysis of a Parallel Networks

(I)

1k		     1k

X

+     +             -       IX     -	+	+

10v	I1     +   -    I2               8v

-                                     4.7k             -             			                     +

-         +

- Theoretically solve for the branch currents for each of the network.

5.7k       -4.7k              I1                     10v		X1=  I1 =  1.865mA

=

-4.7k        5.7k              I2                     -8v		X2=  I2 =  .08542mA

I1=  1.865 mA            IX = I3 = I1 – I2 = (1.865 mA)– (-.1346mA) = 1.865 mA+.1346mA

I3=  1.9996mA             IX = I3 = 1.999mA

I2=  -.1346mA

2. Set up each of the network and measure all branch currents.

I1= I1= 1.783mA

I8v= I2= 0.48mA

I4.7k= I3=1.75mA

Analysis of Parallel Networks

(II)

3.3k		     4.7k	            6.8k

X                                               y

+     +             -       IX     -  	+	         IX   -                   +       +

9v	I1     +   -    I2       +  +                    12v

-                                     2.7k                     1k                             _

-      +                    -	   -

- Theoretically solve for the branch currents for each of the network.

6k      -2.7k      0            I1                    9v		X1=  I1 =  1.646mA

-2.7k    8.4k     -1k          I2         =        0		X2=  I2 =  .3247mA

0        -1k        6.8k        I3                  -12v         X3=  I3 =  -1.717mA

I1=  1.642 mA            IX = I4 = I1 – I2 = (1.684mA ) – (.3247mA) =  1.321mA

I3=  .3247mA             Iy = I5 = I2  – I3 =  (1.769mA) – (-1.717mA) =  2.042mA

I2=  -1.717mA

2. Set up each of the network and measure all branch currents.

I3.3k= I1= 1.583mA

I2.7k= I4= 1.264mA

I4.7k= I2= .319mA

I1k= I5= 2.414mA

I6.8k= I3= -1.735mA

Mesh Analysis for the First Network

|  | Measured | Calculated | %Error |
| --- | --- | --- | --- |
| I1 | 1.783mA | 1.865 mA | 4.6 |
| I2 | -0.48mA | -0.1346mA | 256.6 |
| I3 | 1.75mA | 1.9996mA | 12.48 |

Mesh Analysis for the Second Network

|  | Measured | Calculated | %Error |
| --- | --- | --- | --- |
| I1 | 1.583mA | 1.642mA | 3.6 |
| I2 | 1.264mA | 1.321mA | 4.3 |
| I3 | -.319mA | -.3247mA | -17.55 |
| I4 | 2.414mA | 2.042mA | -18.22 |
| I5 | 1.735mA | 1.717mA | -1.048 |

Conclusion

Mesh analysis is one of the quicker and easier methods to use to find the current(s) of a network.  First of all, to apply this method, I followed mesh’s rules to use the network’s sources.  Second, I divided the networks in meshes to find the current through them. As a result a got simple equations which gave me the currents through the meshes of the networks, and which I used to find the corrents of the resistors shared by each mesh.