Farmingdale State College

Professor:

Subject:

Date:

2008

Manual Lab Experiment Report

Capacitors

Circuit Electronic I

Adam A. Fillios		EET 111		12/18

Made by

Piter Garcia

RC TRANSIENTS

The purpose of this experiment is to study the behavior of an RC charging network.

- Write a program to evaluate any exponential function of the form:

Y (t) = YFinal – (YFinal – YInitial) x e(-tRC)

- Set up the network shown below with a digital voltmeter connected across the capacitor.

Voltimeter

R              ℓc

- R= 10MΩ                                                                                                    +

C= 14µF               20v      	                                  			V		        Rm

-

(b) R= 5MΩ

C= 14µF

Important:

Notice that the final value of Vc will not be equal to Vin = 20 volts.  This happens because the voltmeter connected across the capacitor is in effect an additional resistance R in parallel with the capacitor.

Hence VcFinal = 20 x Rm/ (R + Rm),  time constant τ = Req x C, Where Req = R x Rm/ (R + Rm)

- Taking into account the meter resistances R calculate Vc and Ic in 10 sec intervals for o ≤ t ≥ 500s for each case (a) and (b). Tabulate your results.

- With the capacitor initially uncharged, set up the circuit, close the switch, and measure Vc at the same intervals as in part (3).  Use the measure values of Vc above to find ic for the same values of t.  Tabulate your results.

Development:

Uncharging Circuit:

Voltimeter

R              ℓc

+

20v      	                                  			V		        Rm

-

10MΩ

R Thevenin is going to be equal to 10MΩ. Therefore, the capacitor will be charged with a resistance of 10MΩ, and discharged with a resistance of 5MΩ.

R              ℓc

20v                                       				Rm             5MΩ

Initial Condition:

10MΩ

Ii = E/R= 20v/10MΩ = 2µA

Ii = 2µA                                                                                             R    ℓc	    		                                                                                   τ = Req x C = (10MΩ) (14µF)

τ = 140s                   		       	       Ii	                                           υc

20v

υc = E(1-e(-tRC)) = 20v(1- e(-0140s)) = 20 (1-1)	 	υc = 0

ℓc = Ii(1-e(-tRC)) = 2µA (e(-0140s)) = 2µA (1)	        ℓc = 2µA

υR = E(e(-tRC)) = 20v( e(-0140s)) = 20 (1)	        υc = 20

Development Part (2):

Charging Circuit

10MΩ

Ii = E/R= 20v/10MΩ = 2µA

Ii = 2µA                                                                                             R    ℓc	    		                                                                                   τ = Req x C = (10MΩ) (14µF ) 	 Ii

τ = 140s                   		       	       	                                           υc

20v

t = 10seconds

υc = E(1-e(-tRC) ) = 20v(1- e(-10140s)) = 20 (1-931.1m)	       υc = 1.379v

ℓc = Ii(1-e(-tRC) ) = 2µA (e(-10140s)) = 2µA (931.1m)         ℓc= 1.86µA

υR = E(e(-tRC) ) = 20v(e(-10140s)) = 20 (931.1m)	           υc = 18.62v

t = 500 seconds

υc = E(1-e(-tRC) ) = 20v(1- e(-500140s)) = 20 (1-28.12µ)          υc = 19.44v

ℓc = Ii(1-e(-tRC) ) = 2µA (e(-500140s)) = 2µA (28.12µ)       ℓc= 56.23nA

υR = E(e(-tRC) ) = 20v(e(-500140s)) = 20 (28.12µ)	           υc = .5623v

Development Part (3):

Final Condition

Time Constant: 0≤ T ≥ 5t = ∞

(Open Circuit)

10MΩ

Ii = E/R= 20v/10MΩ = 2µA

Ii = 2µA                                                                                             R          ℓc	    		                                                                                   τ = R x C = (10MΩ) (14µF)                                                                             +                            5MΩ

τ = 140s                   	                                20v 				V

Rthev = 10MΩ            	                                  		 -	    Rm

10MΩ

R    ℓc

Ii				          υc

20v

5t = 5(140) = 700seconds

υc = E(1-e(-tRC)) = 20v(1- e(-700s140s)) = 20 (1-6.730m)          υc = 19.87v	                                                                                                                                     ℓc = Ii(1-e(-tRC)) = 2mA (e(-700s140s)) = 2mA (6.730m)         ℓc= 13.48µA

υR = E(e(-tRC)) = 20v( e(-700s140s))= 20 (6.730m)	 	υR = .1348V

Voltages across the capacitor during the initial and final condition

Vc (20v) _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ 19.87V

υc = E(1-e(-tRC)) = 20v(1- e(-0140s)) = 0v         υc = 20v(1- e(-700s140s))  = 19.9v

0								      5T

Voltages across the capacitor during the initial and final condition

ℓc (2 µA)

ℓc = 2µA ( e(-0140s)) = 2µA

0								      5T

ℓc = Ii(e(-tRC)) = 2mA (e(-700s140s)) = 13.48µA

Voltages across the capacitor during the initial and final condition

Vc (20v) _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ 19.44V

υc = 20v(1- e(-500140s)) =19.44v

υc = E(1-e(-tRC) ) = 20v(1- e(-10140s)) = 1.379v

0 10T								500T

Voltages across the capacitor during the initial and final condition

ℓc (2 µA)

ℓc = Ii(e(-tRC) ) = 2µA (e(-10140s)) = 1.86µA

0 10T					500T

ℓc = Ii(e(-tRC) ) = 2µA (e(-500140s)) = 56.23µA

Development:

R              ℓc

+

20v      	                                  			V		        Rm

-

Initial Condition:

Ii = E/R= 20v/5MΩ = 4µA                                                                                                                                                        Ii = -4µA

τ = R x C = (5MΩ) (14µF)                                                                      ℓc					    τ = 70s

5MΩ

20v            υc		        Rm

υc = E(1-e(-tRC)) = 20v(1- e(-070s)) = 20 (1-1)	     υc = 0

ℓc = Ii(1-e(-tRC)) = -4µA ( e(-070s)) = -4µA (1)	        ℓc = -4µA

υR = E(e(-tRC)) = 20v( e(-070s)) = 20 (1)	         υc = 20v

Development Part (2):

ℓc

Rm             5MΩ

20v                υc

t = 10seconds

υc = E(1-e(-tRC) ) = 20v(1- e(-1070µs)) = 20 (1- 866.9m)	       υc = 2.662v

ℓc = Ii(1-e(-tRC) ) = 4µA (e(-1070s)) = 4µA (866.9m)         ℓc= 3.468µA

υR = E(e(-tRC) ) = 20v(e(-1070s)) = 20 (866.9m)	           υc = 17.34v

t = 500 seconds

υc = E(1-e(-tRC) ) = 20v(1- e(-50070s)) = 20 (1-790.5m)          υc = 19.98v

ℓc = Ii(1-e(-tRC) ) = 4µA (e(-50070s)) = 4µA (790.5m)       ℓc= 3.162nA

υR = E(e(-tRC) ) = 20v(e(-50070s)) = 20 (28.12µ)	           υc = 15.81mv

Development Part (3):

Discharging Circuit

Final Condition: 0≤ T ≥ 5t = ∞

(Open Circuit)

10MΩ

R              ℓc	    		                                                                                   τ = Req x C = (5MΩ) (14µF)                                                                             +                  5MΩ

τ = 70s                   	                                20v 				Vc

-	   			    Rm

ℓc

Rm             5MΩ

υc

5t = 5(70) = 350seconds

υc = E(1-e(-tRC)) = 20v(1- e(350-s70s)) = 20 (1-6.730m)          υc = 19.87v	                                                                                                                                     ℓc = Ii(1-e(-tRC)) = 2mA (e(350-s70s)) = 2mA (6.730m)         ℓc= 40.43µA

υR = E(e(-tRC)) = 20v( e(-350s70s))= 20 (6.730m)	 	υR = .1348V

Voltages across the capacitor during the initial and final condition

Vc (20v)

19.87V

υc = E(1-e(-tRC)) = 20v(1- e(-700140s)) = 19.87v

5T

Voltages across the capacitor during the initial and final condition

ℓc (4 µA)

ℓc = 4µA (1- e(-0140s)) = 4µA

5T

ℓc = Ii(1-e(-tRC)) = 2mA (e(-700s140s)) = -40.43µA

Voltages across the capacitor during the initial and final condition

Vc (20v) _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ 19.8V

υc = 20v(1- e(-500140s)) =19.8v

υc = E(1-e(-tRC) ) = 20v(1- e(-10140s)) = 2.662v

0 10T								500T

Voltages across the capacitor during the initial and final condition

ℓc (2 µA)

ℓc = Ii(1-e(-tRC) ) = 4µA (e(-10140s)) = 3.468µA

0 10T					500T

ℓc = Ii(1-e(-tRC) ) = 4µA (e(-500140s)) = 3.162nA

The content of this experiment is based on capacitors.  A capacitor is made by two plates that share neutrons and electrons one another; its function consists on holding the voltage until it gets its maximum or final voltage in a network.  The experiment above was made through the following steps: First, I leveled the voltage and capacitor to zero applying short circuits.  I got the Tau (t), time constant in which the capacitor is going to be charged or discharged.  Then, I initial was obtained dividing the voltage or Thevenin voltage of the network with the Thevenin resister.  Second, the next formula:

(Vc= Vf + (Vi –Vf) e-tT)

It was applied to get the voltage across the capacitor.  However, since there is no initial voltage, the formula which I worked with is:

(Vc= Vf (1 – e-tT)

Once I wrote the mathematical expressions for the voltage and the current across the capacitor and the resistor, I started solving the circuit with the Initial condition in which time is equal zero.  Third, following the steps from the experiment in which 10 and 500 seconds were given as time, I found the initial current and voltage across the capacitor and the resistor during certain time.  Finally, I applied the discharging cycle or final condition in which 0≤ 5T ≥ ∞ and the circuit is acting as an open circuit.