To experimentally observe and analyze the performance of a zener regulated linear power supply in terms of its regulation, ripple and ripple rejection characteristics. The operation will be examined with and without an operational amplifier in the controlling feedback loop and the performance variations will be noted.

1.

Oscilloscope

2.

Half, Full Center Tap, and Full Wave Rectifier Board

3.

Ohmmeter (Fluke – 45)

4.

Powerstat-Variable Transformer 3 Amp

5.

2 Capacitors – 100 μfd and

6.

Resistor - Pot

7.

Various lengths of hook-up wire and instrument leads as required

- Low line no load measurements (Variac Voltage = 105):

- V out-dc= 4.427V

- I Load= 0

- V-out-pp-ripple = 5.61mV

- V zener = 4.996V

- V zener-pp-ripple =  5mV

- V cf = 16.239V

- V cf-pp-ripple = 2.35V

- High line no load measurements (Variac Voltage = 120):

- V out-dc= 4.433V

- I Load= 0

- V-out-pp-ripple = 5.94mV

- V zener = 5.005V

- V zener-pp-ripple = 6.6mV

- V cf = 19.04V

- V cf-pp-ripple = 2.71V

- Low line full load measurements (Variac Voltage = 105 / R-load = 100 ohms):

- V out-dc= 3.953V

- I Load=  39.53mA

- V-out-pp-ripple = 5.9mV

- V zener = 4.984V

- V zener-pp-ripple = 26mV

- V cf = 13.978V

- V cf-pp-ripple = 7.85V

- High line full load measurements (Variac Voltage = 120 / R-load n= 100):

- V out-dc= 3.948V

- I Load= 39.48mA

- V-out-pp-ripple = 5.6mV

- V zener = 4.992V

- V zener-pp-ripple = 23.2mV

- V cf = 16.3V

- V cf-pp-ripple = 8.30V

Performance parameters for the power supply obtained from part (a) through        (d) with the above calculations.

- The output % line regulation at no load:

For Vcf

16.3v- 13.978v16.3v*100=14.25

For Vcf

16.239v-13.978v16.239v*100=13.92

For Vcf

19.04v-16.3v19.04v*100=14.37

For Vout

4.433v-4.427v4.433v*100= .14

For  Vcf

19.04v-16.239v19.04v*100=14.69

- The output % line regulation at full load:

For Vout

3.953v-3.948v3.953v*100= .13

- The output % load regulation at low line:

For Vout

4.958v-4.427v4.958v*100=11.18

- The output % load regulation at high line:

For Vout

4.433v-3.948v4.433v*100=10.94

For LL-NL    20log(10525m ) = 90db

The ripple rejection in db from input to the zener:

For HL-NL    20log(12026.6m ) = 88db

For LL-NL    20log(5m5.61m ) = -1db

For HL-NL    20log(6.6m5.94m ) = 1db

The ripple rejection in db from the zener to the output

For HL-NL    20log(12025.94m ) = 88db

For LL-NL    20log(10525.61m ) = 90db

The ripple rejection in db from input to output:

| Output % Line and Load Regulation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Load | Line Regulation |  | Load Regulation | Ripple Voltage | Ripple Rejection in db |  |
|  |  |  |  |  | LL | HL |
| None | .14 | LL | 11.18 | V cfpp | 90db | 88db |
| Full | .13 | HL | 10.94 | V zenerpp | 1db | 1db |
|  |  |  |  | Voutpp | 90db | 88db |

- Low line no load measurements (Variac Voltage = 105):

- V out-dc= 6.084V		VR6= 6.26v  		R6= 3k

- I Load= 0

- V-out-pp-ripple = 2.568V

- V zener= 4.998V

- V zener-pp-ripple = 230μV

- V cf = 16.7V

- V cf-pp-ripple = 2.56V

- High line no load measurements (Variac Voltage = 120):

- V out-dc= 6.473V

- I Load= 0

- V-out-pp-ripple = 2.94V

- V zener = 5V

- V zener-pp-ripple = .210mV

- V cf = 18.255V

- V cf-pp-ripple = 1.01V

- Low line full load measurements (Variac Voltage = 105 / R-load = 100 ohms):

- V out-dc= 8.519V

- I Load= 85.1mA

- V-out-pp-ripple = 9.99V

- V zener= 4.968V

- V zener-pp-ripple = 62.9mV

- V cf = 10.840V

- V cf-pp-ripple = 11.14V

- High line full load measurements (Variac Voltage = 120 / R-load n= 100):

- V out-dc= 10.085V

- I Load= 100.85mA

- V-out-pp-ripple = 11.83V

- V zener = 4.977V

- V zener-pp-ripple = 53mV

- V cf = 12.546V

- V cf-pp-ripple = 13.18V

Performance parameters for the power supply obtained from part (a) through (d) with the above calculations.

- The output % line regulation at no load:

For Vout

6.473v-6.084v6.473v*100=6.01

For  Vcf

18.255v-16.7v18.255v*100=8.52

100=6.01

- The output % line regulation at full load:

For Vout

10.085v-8.519v10.085v*100=15.53

For Vcf

12.54v- 10.84v12.54v*100=13.56

- The output % load regulation at low line:

For Vcf

16.7v-10.84v16.7v*100=  35.9

For Vout

8.519v-6.084v8.519v*100=28.58

- The output % load regulation at high line:

For Vout

10.085v-6.473v10.085v*100=35.82

For Vcf

18.255v-12.54v18.255v*100=31.31

- The ripple rejection in db from input to the zener:

For HL-NL   20log(1202.210m ) = 118db

For LL-NL  20log(.230m2.5681m ) = -81db

For LL-NL    20log(1052.230m ) = 96db

For HL-NL    20log(12022.94 ) = 35db

For LL-NL    20log(10522.568 ) = 35db

For HL-NL    20log(.210m2.94 ) = 83db

- The ripple rejection in db from the zener to the output

- The ripple rejection in db from input to output:

| Output % Line and Load Regulation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Load | Line Regulation |  | Load Regulation | Ripple Voltage | Ripple Rejection in db |  |
|  |  |  |  |  | LL | HL |
| None | 6.01 | LL | 28.58 | V cfpp | 96db | 35db |
| Full | 15.53 | HL | 35.82 | V zenerpp | -81db | 83db |
|  |  |  |  | Voutpp | 35db | 118db |

| Tabulated Calculations (1): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Output % Line and Load Regulation |  |  |  |  |  |
| Load | Line Regulation | Load Regulation | Ripple Voltage | Ripple Rejection in db |  |
| None | .14 | 11.18 |  | LL | HL |
| Full | .13 | 10.94 | V cfpp | 90db | 88db |
|  |  |  | V zenerpp | 1db | 1db |
|  |  |  | Voutpp | 90db | 88db |
| Tabulated Calculations (2): |  |  |  |  |  |
| Output % Line and Load Regulation |  |  |  |  |  |
| Load | Line Regulation | Load Regulation | Ripple Voltage | Ripple Rejection in db |  |
| None | 6.01 | 35.82 |  | LL | HL |
| Full | 15.53 | 28.58 | V cfpp | 96db | 35db |
|  |  |  | V zenerpp | -81db | 83db |
|  |  |  | Voutpp | 35db | 118db |