To experimentally investigate the operation of a simulated switching power supply (DC  to DC converter) and compare experimental results to theoretical expectations and to results predicted by a simple PSPICE simulation.

1.

Oscilloscope

2.

Half, Full Center Tap, and Full Wave Rectifier Board

And a XFMR board

3.

Ohmmeter (Fluke – 45)

4.

Powerstat-Variable Transformer 3 Amp

5.

1 Capacitors – 250 μfd and a Zener diode

6.

Resistor - Pot

7.

Various lengths of hook-up wire and instrument leads as required

Refer to figure #1 and assume the following nominal requirements:

VIN=12 volts dc	V-output= 6 volts dc		V-diode- 0.6v

VCESAT- 0.5V	f-operating= 40k

Using the above specifications and component values shown in fig. 1 obtain the following. (Show all work)

- The Inductor current waveform and the maximum ∆I expected for I-out=100mA.

Current Waveform

Maximum ∆I expected

∆I= 1L*Vin-Vcesat-Vd*ton

∆I= 1500µ*12-0.5-0.6*ton

ton= kd*t1+kd         kd=(Vo+Vd)Vin-Vcesat-Vo	       ton=(Vo+Vd)*tVin-Vcesat-Vo Vin-Vcesat-VoVin-Vcesat-Vd

ton= (Vo+Vd)Vin-Vcesat+Vd = (6+0.6)12-0.5+0.6 = 13.636 µs				ton=13.636 µs

∆I= 1500µ*12-0.5-0.6*13.636 µs = 297.27mA		∆I=297.27mA

- The expected t-on and t-off for the following output voltage  values:

Vout=4, and 8. (VIN=12)

ton= kd*t1+kd         kd=(Vo+Vd)Vin-Vcesat-Vo	       ton=(Vo+Vd)*tVin-Vcesat-Vo Vin-Vcesat-VoVin-Vcesat-Vd

For Vout=4 volts

ton= (Vo+Vd)Vin-Vcesat+Vd = (4+0.6)12-0.5+0.6 = 9.504 µs				ton=9.429 µs

t= 140k = 25 µs         toff= 25 µs - 9.504 µs = 15.5 µs			toff=15.5 µs

For Vout=4 volts

ton= (Vo+Vd)Vin-Vcesat+Vd = (8+0.6)12-0.5+0.6 = 17.769 µs				ton=17.77 µs

t= 140k = 25 µs         toff= 25 µs - 17.769 µs = 7.23 µs		toff=7.23 µs

- The expected t-on and t-off for the following input voltages  :

Vin=10, and 14. (V-out=6)

For Vin=10 volts

ton= (Vo+Vd)Vin-Vcesat+Vd = (6+0.6)10-0.5+0.6 = 21.287 µs				ton=21.29 µs

t= 140k = 25 µs         toff= 25 µs - 21.287 µs = 3.713 µs		toff=3.713 µs

For Vin=14 volts

ton= (Vo+Vd)Vin-Vcesat+Vd = (6+0.6)14-0.5+0.6 = 15.248 µs				ton=15.25 µs

t= 140k = 25 µs         toff= 25 µs - 15.248 µs = 9.752 µs		toff=9.75µs

- The voltage waveform across the flyback diode.

- The voltage waveform across the switching transistor.

- The input current waveform and its average dc value.

I-in dc= Io*tont

I-in dc= 100mA*13.636 µs25 µs  = 54.545mA	 I-in dc= 54.55mA

∆I= 1L*Vin-Vcesat-Vd*ton

∆I= 1500µ*12-0.5-0.6*13.636 µs = 297.27mA	 ∆I=297.27mA

- The input dc power supplied by input and the % efficiency.

P-inputAve= I-in dc*Vin

P-in= 54.55mA*12 = 655mW				P-in= 655mW