The objective of this experiment is to study through experimentation the basic characteristics of a simulated resistive transducer bridge and the associated instrumentation amplifier. Experimental results will be checked against theoretical expectations.

1.

Proto – Board, Model 203A

2.

M4741 Operational Amplifier

3.

Ohmmeter

7.

Various lengths of hook-up wire and instrument leads as required

Vin

V-out

Vin1= Vin (RBRA+RB)

Vin2= Vin (RCRC+Rpot)

Vout= (Vin2-Vin1) * RfR1

RA=RB=RC= 4.99k

Rp-Max= 8.703k

V-out at R-pot=0

Vin1= 5v (4.99k4.99k+4.99k) = 2.5v

Vin1=2.5v

Vin1 will always remain the

same since its resistance is

not being affected by the Rpot.

Vin2= 5v (4.99k4.99k+0) = 5 v

V-out= (2.5-5) * 50k10k= -12.5

Vout= -12.5

V-out at R-pot=200

Vin1=2.5v

Vin2= 5v (4.99k4.99k+200) = 4.807v

Vout= (2.5-4.81)* 51 = -11.537v

Vout= -11.537v

V-out at R-pot=1k

Vin1=2.5v

Vin2= 5v (4.99k4.99k+1k) = 4.165v

Vin2=4.17v

Vout= (2.5-4.17)* 51 = -8.326v

Vout= -8.33v

R-pot at V-out = 0v

Vin1=2.5v

∴ Vin2 = 2.5 = 5v (4.99k4.99k+Rp)

(4.99k + Rp) = 5v (4.99k4-2.5)

Rp= 9.98k – 4.99k = 4.99k

Rp= 4.99k

R-pot at V-out = -12.5v

Vin1=2.5v

∴ Vin2 = 5 = 5v (4.99k4.99k+Rp)

(4.99k + Rp) = 5v (4.99k5)

Rp= 4.99k – 4.99k = 0

Rp= 0

R-pot at V-out = -5v

Vin1=2.5v

∴ Vin2 = 3.5 = 5v (4.99k4.99k+Rp)

(4.99k + Rp) = 5v (4.99k3.5)

Rp= 7.129k – 4.99k = 2.139k

Rp= 2.14k

R-pot at V-out = 2.5v

Vin1=2.5v

∴ Vin2 = 2 = 5v (4.99k4.99k+Rp)

(4.99k + Rp) = 5v (4.99k2)

Rp= 12.475k – 4.99k = 7.485k

Rp= 7.49k

V-out at R-pot=4k

Vin1=2.5v

Vin2= 5v (4.99k4.99k+4k) = 2.775v

Vin2=2.78v

Vout= (2.5-2.78)* 51 = -1.377v

Vout= -1.38v

V-out at R-pot=5k

Vin1=2.5v

Vin2= 5v (4.99k4.99k+5k) = 2.498v

Vin2= 2.498v

Vout= (2.5-2.498)* 51 = 0.0125

Vout= 0.013v

V-out at R-pot=6k

Vin1=2.5v

Vin2= 5v (4.99k4.99k+6k) = 2.27v

Vin2= 2.498v

Vout= (2.5-2.27)* 51 = 1.149

Vout= 1.145

V-out at R-pot= R-Max

Vin1=2.5v

Vin2= 5v (4.99k4.99k+8.703k) = 1.822v

Vin2=1.82v

Vout= (2.5- 1.82)* 51 = 3.389v

Vout= 3.39v

| Measuring the Output Voltage In terms of R-pot | Measuring the R-pot In terms of Output Voltage |
| --- | --- |
| V-out at R-pot=282 Vout= -12.968v V-out at R-pot=194 Vout= -11.939v V-out at R-pot=1.004k Vout= -8.640v V-out at R-pot=4.003k Vout= -1.38v V-out at R-pot=5k Vout= 0.013v V-out at R-pot=6.018k Vout= 1.175 V-out at R-pot= 8.703k Vout= 3.489 | R-pot at V-out = 0.005v Rp= 5.022k R-pot at V-out = -12.558v Rp= 0.083 R-pot at V-out = -5.016v Rp= 2.210k R-pot at V-out = 2.504v Rp= 7.406k |

| Output Voltage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Theoretical | Experimental | % Error |
| R-pot | Vin1 | Vin2 | Vout | Vout |  |
| 0 Ω | 2.5v | 5 v | -12.5v | -12.968v | 3.72 |
| .2k Ω | 2.5v | 4.807v | -11.537v | -11.939v | 3.48 |
| 1k Ω | 2.5v | 4.165v | -8.33v | -8.640v | 3.74 |
| 4k Ω | 2.5v | 2.78v | -1.38v | -1.38v | 0 |
| 5k Ω | 2.5v | 2.498v | 0.013v | 0.013v | 0 |
| 6k Ω | 2.5v | 2.27v | 1.145v | 1.175v | 2.62 |
| 8.703k Ω | 2.5v | 1.822v | 3.39v | 3.489v | 2.92 |
| R-pot Resistance |  |  |  |  |  |
|  |  |  | Theoretical | Experimental | % Error |
| Vout | Vin1 | Vin2 | R-pot | R-pot |  |
| 0v | 2.5v | 2.5v | 4.99k Ω | 5.022k Ω | 0.64 |
| -12.5v | 2.5v | 5v | 0 Ω | 0.083 Ω | 0 |
| -5v | 2.5v | 3.5v | 2.14k Ω | 2.210k Ω | 3.27 |
| 2.5v | 2.5v | 2v | 7.49k Ω | 7.406k Ω | 1.12 |

Using your experimental graph of Rpot Vs Vout and some calculations determine:

Rth=Rpot is 8k at 20°C.

- Vout at 20°C

Vin1=2.5v

Vin2= 5v (4.99k4.99k+8k) = 1.9207v

Vin2=1.82v

Vout= (2.5- 1.921)* 51 = 2.897v

Vout= 2.9v

Vin1=2.5v

Vin2= 5v (4.99k4.99k+3k) = 3.1226

Vin2= 3.123v

Vout= (2.5- 3.123)* 51 = -3.113v

Vout= -3.113v

- Vout at 30°C

For 30°C ∆T = 10°C

∆Rth= Tc * ∆T

∆Rth= .5k°C  * 10°C  = -5k

Rth= ∆Rth + Rth = -5k + 8k = 3k

- The Temperature when Vout is  zero volts

Rth= Tc * ∆T  ∆T =(∆RthTC) = (-3.01k-.5K)

∆T = 6.02°C  T= 26.02°C

(RCRth) = (RBRA) 	Rth = (RC*RARB)

∆Rth = (RC*RARB) = 4.99K

Rth= 4.99K – 8k = -3.01k

Rth= Tc * ∆T   ∆T = (∆RthTC) = (-8k-.5K)

∆T = 16°C   T= 36°C

Temperature whe Vout is -12.5

R-pot at V-out = -12.5v

Vin1=2.5v

∴ Vin2 = 5            Rpot = 0

For ∆Rth= Rp= 0 Rth must be = 8k

∆Rth= Tc * ∆T

In the experiment above we learned how to measure the R-pot in its different points of references from the minimum to the maximum values. We also observed the V-output response to the changes in the adjustable resistance (R-pot) of the network; from which we can conclude that, the greatest is the R-pot applied to the circuit the greatest is the value of the V-output.