The purpose of this experiment is to illustrate that the principle of superposition can be applied to circuits with dependent and independent sources.

The superposition theorem for electrical circuits states that the response, voltage or current, in any branch of a bilateral linear circuit having more than one independent source equals the algebraic sum of the responses caused by each independent source acting alone, while all other independent sources are replaced by their internal impedances.

The principle of additivity can be explained as following. When there are two sources in a circuit, the contribution due to each source can be found out separately and the response due to both sources is the algebraic sum of contributions due to each source acting alone. This aspect is illustrated by the circuit shown below. The current through the resistors is the sum of currents due to sources of node 1 and the node 3, acting alone. When the contribution due to node 1 is to be calculated, the contribution due to the source in node 3 should be zero. Similarly, source in node 1 can be replaced by an open circuit, the contribution due to the source in node 3 is to be calculated, and the contribution due to the source in node 1 should be zero. The relevant circuits are shown below.

4)

D) Verify the principle of superposition, by adding the individual responses (obtained in parts b and c) and comparing them with the total responses (obtained in part a)

| Experimental Table Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | V1 | V2 | V3 | V4 | V5 |
| B | 58 | 55 | 0 | 65 | -15 |
| C | 20 | 20 | 20 | 20 | 0 |
| B+C | 78 | 75 | 20 | 85 | -15 |
| A | 78 | 75 | 20 | 85 | -15 |

5)

| Table Data for Current Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5a | V1 | V2 | V3 | V4 | V5 |
|  | 58 | 55 | 0 | 65 | -15 |

I1= 1mA		V3=0

VR1= R1*I1= 3k*1mA= 3v

VR1= 3v

VR3= R3*10*I1= 1k*10mA

VR3= 10v

V1= VR1 + VR2= 3v + 55v= 58v

V1= 58v

V2= VR2= 55v

V2=55v

V3=0

V1, 2= VR1= 3v

V2, 3= VR2= 55v

VR2= R2*(I1+ 10*I1)

VR2= 5K*(1mA+10mA)= 55v

VR2= 55v

VR4= R4*10*I1= 1.5K*10mA

VR4= 15V

V4= VR3+VR2= 10v + 55v= 65v

V4= 65v

V5= -VR4= -15v

V5= -15v

| Table Data for Voltage Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | V1 | V2 | V3 | V4 | V5 |
| 5b | 20 | 20 | 20 | 20 | 0 |

I1= 0		V3= 20

VR1= R1*I1= 3k*0mA= 0v

VR1= 0v

VR3= R3*10*I1= 1k*0mA

VR3= 0v

V4= VR3+VR2 -20 =

V4= 0v + 0v -20 = 20v

V4= 20v

V5= -VR4= -0v

V5= 0V

VR2= R2*(I1+ 10*I1)

VR2= 5K*(0mA+0mA)= 0v

VR2= 0v

VR4= R4*10*I1= 1.5K*0mA

VR4= 0v

V1= VR1 + VR2= 0v + 0v -20

V1= 20v

V2= VR2 - 20= 0 - 20

V2=20v

V3= 20

V1, 2= VR1= 0v

V2, 3= VR2= 0v

| Theoretical Table Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | V1 | V2 | V3 | V4 | V5 |
| 5a | 58 | 55 | 0 | 65 | -15 |
| 5b | 20 | 20 | 20 | 20 | 0 |
| 5a+b | 78 | 75 | 20 | 85 | -15 |

| Experimental Table Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | V1 | V2 | V3 | V4 | V5 |
| 5a | 58 | 55 | 0 | 65 | -15 |
| 5b | 20 | 20 | 20 | 20 | 0 |
| 5a+b | 78 | 75 | 20 | 85 | -15 |

| Comparison Table Between Both Tables |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | V1 | V2 | V3 | V4 | V5 |
| Theoretical | 78 | 75 | 20 | 85 | -15 |
| Experimental | 78 | 75 | 20 | 85 | -15 |
| % Error | 0 | 0 | 0 | 0 | 0 |

In the experiment above, the principles of superposition were proved in the following ways: First, to ascertain the contribution of each individual source, all of the other sources first must be "turned off or set to zero by:

First, replacing all other independent voltage sources with a short circuit, thereby eliminating difference of potential V=0, internal impedance of ideal voltage source is zero because is a short circuit.

Second, replacing all other independent current sources with an open circuit , thereby eliminating current I=0, internal impedance of ideal current source is infinite because it is an open circuit.

This procedure was followed for each source in turn, and then the resultant responses were added to determine the true operation of the circuit. The resultant circuit operation is the superposition of the various voltage and current sources.