Name: Piter Garcia

Electrical Engineering Technology

Experiment No. 6

Unregulated Power Supply Measurements

Date of Experiment:

03/29/09

Date handed in:

03/29/09

Date due:

Grade:

Lab Partners:

Khiry Marshall

2009

Farmingdale State College State University of New York

PedritoGV

[Type the company name]

3/29/2009

Objectives:

The objective of this experiment is to understand how proportional the output DC voltage is to the AC input voltage.  In other words, to study how the output voltage behaves in step with input variations as well as changes in load impedance.

| Item# | Description | Manufacture | Model# | Serial# |
| --- | --- | --- | --- | --- |
| 1 | Fluke 45 Dual Display Multimeter | RACAL | 1 | 02367 |
| 2 | Keithly TRMS Multimeter | Keithly | 170A | 02260 |
| 3 | Decade Boxes | --- | --- | --- |
| 4 | Transformer | --- | --- | --- |
| 5 | Oscilloscope PM 3215 50mhz | Philips | 241-11 | 02269 |

Calculations

Part B:

Step# 1: IL-DC = VRS/10

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | 200v10 | 20mA |
| 160v | 160v10 | 16mA |
| 120v | 120v10 | 12mA |
| 80v | 80v10 | 8mA |
| 40v | 40v10 | 4mA |
| 0v | 0v10 | 0mA |

Step#2: VL-DC= VOP-Vr/2

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | 8.2v - .6v2 | 8.61v |
| 160v | 8.1v - 410v2 | 6.701v |
| 120v | 8.15v - .4v2 | 8.711v |
| 80v | 8.2v - .204v2 | 8.81v |
| 40v | 8.25v- .121v2 | 8.85v |
| 0v | 9.2v - 0v2 | 8.91v |

Step#3: %Error

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | ( (8.61-7.956)7.956 ) (100) | 8.17% |
| 160v | ( (8.701-8.09)8.09 ) (100) | 7.40% |
| 120v | ( (8.711-8.4)8.4 ) (100) | 3.57% |
| 80v | ( (8.81-8.61)8.61 ) (100) | 2.33% |
| 40v | ( ( 8.85 - 8.79)8.79 ) (100) | .57% |
| 0v | ( (9.38-8.91)9.38 ) (100) | 4.8% |

Step#4: Vr= IL – DC * Toff/CL

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | ( 20mA (14.2ms)471µfd ) | .6v |
| 160v | ( 16mA (14.4ms)471µfd ) | .49v |
| 120v | ( 12mA (14.6ms)471µfd ) | .372v |
| 80v | ( 8mA (14.8ms)471µfd ) | .251v |
| 40v | ( 4mA (15.4ms)471µfd ) | .131v |
| 0v | ( 0mA (0ms)471µfd ) | 0v |

Step#5: %Error

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | ( (.6v-.6v).6v ) (100) | 0% |
| 160v | ( (.49v-.410v)4.10v ) (100) | 19.5% |
| 120v | ( (.372v-.4v).4v ) (100) | 7% |
| 80v | ( (.251v-.204v).204v ) (100) | 23% |
| 40v | ( .131v - ,121).121 ) (100) | 8.01% |
| 0v | ( (0v-0v)0v ) (100) | 0% |

Step#6: Vr-rms= Vrp-p/(23)

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | ( .6v23 ) | .173v |
| 160v | ( .49v23 ) | .142v |
| 120v | ( .372v23 ) | .107v |
| 80v | ( .251v23 ) | .0725v |
| 40v | ( .131v23 ) | .0378v |
| 0v | ( .0v23 ) | 0v |

Step#7: %Error

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | ( (.173v- .179v).179 ) (100) | 2.8% |
| 160v | ( (.142v-.149v).149v ) (100) | 4.5% |
| 120v | ( (.107v-.114v).114v ) (100) | 6.06% |
| 80v | ( (.0725v-.077v).077v ) (100) | 6.1% |
| 40v | ( .0378- ,039).039 ) (100) | 2.56% |
| 0v | ( (0v-0v)0v ) (100) | 0% |

Step#8: VL-Total RMS= DC2+AC2

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | 7.9562+.17872 | 7.86v |
| 160v | 8.092+.1492 | 8.09v |
| 120v | 8.42+.11392 | 8.4v |
| 80v | 8.592+..0772 | 8.56v |
| 40v | 8.792+.0392 | 8.79v |
| 0v | 9.382+02 | 9.38v |

Step#7: %Error

| Volts | Calculation | Result |
| --- | --- | --- |
| 200v | ( (7.96v- 7.86v)7.86 ) (100) | 1.26% |
| 160v | ( (8.104v-8.09v)8.09v ) (100) | .12% |
| 120v | ( 8.4v-8.4v)8.4v ) (100) | 0% |
| 80v | ( (8.6v-8.56v)8.6v ) (100) | .47% |
| 40v | ( 8,79- 8.78)8.78 ) (100) | .11% |
| 0v | ( (9.38v-9.36v)9.38v ) (100) | .21% |

Through the information obtained from the measured and calculated data, I conclude that the DC Voltage’s behavior is dependent upon an internal voltage reduction transformer.  This increases by a small amount, to near the peak value of the ripple voltage, less than a volt.  In addition, the output DC Voltage increases as the current provided by the adapter output to the load decrease.  It is a fact that all variations produced in the network are related to the amount of current used by the electrical load.