Part I

| Vin-pk | fs (Hz) | Vo-pk | fs (Hz) |
| --- | --- | --- | --- |
| 4.5 | 948 | 13.1 | 948 |

3) Record V-out and V1 on the scope in volts peak. Also record the frequency of V-out with a counter.

4) Finally record the value of the decade box that gave stable oscillation.

Rf= 43.45k

Part II

- From the decade box setting in part 1 what is the amplifier gain?

- What is the Amplifier gain from the ratio of V-out and V1?

- From Class notes and Text what type of oscillator is this?

- What is theoretical frequency of oscillation? What is the % error?

The experiment above is a Wien Bridge Oscillator. This has both negative and positive feedback. Rf and R1 provide negative feedback and set the forward gain for the positive feedback system. On the other hand, the positive feedback is from output to the V1 through a series RC || RC, lead – lag network.