SB-400 FREQ. PLAN

When the audio signal from V1B and the RF signal from carrier generator cathode follower V2c are applied to the 4-diodes balanced modulator, two different frequencies are produced. These signals are the sum and differences between the audio and carrier frequencies. Neither the audio or carrier signal appears in the output, which is the signal on the air desired to be USB. Therefore, I picked the LSB signal because a component which flips the sideband orientation relative to the carrier.

The output signal of the driver stage is coupled through capacitor C118 to the grids of the final ampliers, in this instant at a frequency of 3.3936 MHz.

The LMD, consisting of tube V6 with its associated circuit, is a very stable linear oscillator that operates over a frequency range of 5 to 5.5 MHz.

The LSB generator (3.3936 MHz) and of LMO mixer V4 are mixed in the tube to produce the sum and difference of two frequencies. The output Sideband and signal from V4 is coupled through the band-pass coupler to heterodyne mixer V5. The band-pass coupler is tuned to pass only signal with a frequency between 8.396 MHz and 8.895 MHz. Only the difference of the LSB generator and LMO signal frequencies fall within this frequency range.

Band Switch wafer 1F selects the proper plate coil for the desired frequency operation. The output of V8A is coupled through capacitor C52 and C51 to the grid of heterodyne mixer V5.

The Crystal filter is 37.395MHZ, designed to pass the usable frequency 3.3936MHZ (LSB) modulation signal.  In this case, only the difference frequencies are permitted to pass through, all other frequencies are passed to ground.  In this instance, the difference between the Sideband frequency of 8.395MHZ-8.895MHZ and the Crystal frequency 37.395 results in an output frequency of 28.5MHZ on the air.

Given Crystal Frequencies:

F1= 12.395 MHZ

F2= 15.895 MHZ

F3= 22.895 MHZ

F4= 29.895 MHZ

F5= 36.895 MHZ

F6= 37.395 MHZ

F7= 37.895 MHZ

F8= 38.395 MHZ

Calculation Example

Signal on the air = Sa

Difference= Diff

Band Pass Filter= BPF

Diff-BPF= Sa 		Diff= Sa +BPF

Diff= 28.85MHZ + 8.395MHZ= 37.245MHZ

Diff=28.85MHZ + 8.895MHZ=37.695MHZ

∴ Crystal Frequency ≅ F6=37.395 MHZ

USB Signal Output Rate:

37.395 MHZ -8.395MHZ = 28.5MHZ

37.395 MHZ -8.895MHZ= 29MHZ