Theoretical Data

Part A: The phase of the angle can be calculated by:

Phase angle =  tT x 360°

Frequency (Hz)  	   Phase Angle

F= 200

Development:

T= 1/200 = 5ms  	.804ms5ms x 360° = 57.86°			P     ∅ = 57.86°

t = .8036ms

F= 300

Development:

T= 1/300 = 3.33ms

t = .432ms                        .432ms3.33ms x 360° = 46.7°		P     ∅ = 46.7°

F= 400

Development:

T= 1/400 = 2.5ms  	.267ms2.5ms x 360° = 38.5°			P     ∅ = 38.5°

t = .267ms

F= 500

Development:

T= 1/500 = 2ms

t = .181ms                        .1806ms2ms x 360° = 32.5°		P     ∅ = 32.5°

F= 600

Development:

T= 1/600 = 1.667ms  	.1294ms1.667ms x 360° = 27.95°			P     ∅ = 27.95°

t = .1294ms

F= 800

Development:

T= 1/800 = 1.25ms

t = .0754ms                        .0754ms1.25ms x 360° = 21.7°		P     ∅ = 21.7°

F= 1000

Development:

T= 1/1000 = 1ms  	.049ms1ms x 360° = 17.65°			P     ∅ f= 17.65°

t = .049ms

F= 1200

Development:

T= 1/1200 = .833ms  	.0345ms.833ms x 360° = 14.9°			P     ∅ = 14.9°

t = .0345ms

F= 1500

Development:

T= 1/1500 = .667ms  	.022ms.667ms x 360° = 11.98°			P     ∅ = 11.98°

t = .022ms

Theoretical Data

Part B: The phase of the angle can be calculated by:

Xc=12π f c 	Phase      ∅ = tanh-1(Xc/R)

R = 500

Frequency (Hz)  	   Phase Angle

F= 200

Development:

Xc = 1/(2π* 200*1µ)       	tanh-1795.775500 = 57.86°	   P     ∅ = 57.86°

Xc = 795.775

F= 300

Development:

Xc = 1/(2π* 300*1µ)       	tanh-1530.516500 = 46.7°	   P      ∅ = 46.7°

Xc = 530.516

F= 400

Development:

Xc = 1/(2π* 400*1µ)		 tanh-1397.887500 = 46.7° 	   P	∅= 38.5°

Xc = 397.887

F= 500

Development:

Xc = 1/(2π* 500*1µ)		 tanh-1318.31500 = 32.5°	   P	∅= 32.5°

Xc = 318.31

F= 600

Development:

Xc = 1/(2π* 600*1µ)		 tanh-1265.258500 = 27.95°		P     ∅= 27.95°

Xc = 265.258

F= 800

Development:

Xc = 1/(2π* 800*1µ)		 tanh-1198.944500 = 21.7°		P     ∅= 21.7°

Xc = 198.944

F= 1000

Development:

Xc = 1/(2π*1000*1µ)		 tanh-1159.155500 = 17.5°		P     ∅= 17.5°

Xc = 159.155

F= 1200

Development:

Xc = 1/(2π*1200*1µ)		 tanh-1132.629500 = 21.7°		P     ∅= 14.9°

Xc = 132.629

F= 1500

Development:

Xc = 1/(2π*1500*1µ)		 tanh-1106.103500 = 11.98°		P     ∅= 11.98°

Xc = 106.103

Theoretical Data

Table A: The percent of errors:

Frequency (Hz)  	   Error Percent

F= 200

Development:

Angle Measured = 50

Angle Calculated = 57.86

Ep = 57.86-5050(100) = 15.72		        Ep = 15.7%

F= 300

Development:

Angle Measured = 40

Angle Calculated =46.7     Ep = 46.7-4046.7(100) = 3.64026		Ep = 3.6%

F= 400

Development:

Angle Measured = 43.2

Angle Calculated = 38.5

Ep = 43.2-38.538.5(100) = 12.2078		Ep = 12.2%

F= 500

Development:

Angle Measured = 36

Angle Calculated = 32.5

Ep = 36-32.532.5(100) = 10.7692		Ep = 10.8%

F= 600

Development:

Angle Measured = 22.5

Angle Calculated = 27.95

Ep = 27.95-22.527.95(100) = 19.4991		Ep = 19.5%

F= 800

Development:

Angle Measured = 17.4

Angle Calculated = 21.7

Ep = 21.7-17.421.7(100) = 19.8157		Ep = 19.8%

F= 1000

Development:

Angle Measured = 14.4

Angle Calculated = 17.65

Ep = 17.65-14.417.65(100) = 14.7651		Ep =14.77%

F= 1200

Development:

Angle Measured = 17.1

Angle Calculated = 14.9

Ep = 17.1-14.914.9(100) = 15.5807		Ep = 15.6%

F= 1500

Development:

Angle Measured = 18

Angle Calculated = 11.98

Ep = 18-11.9811.98(100) = 50.2504		Ep = 50.3%

Theoretical Data

Table A: The percent of errors:

Frequency (Hz)  	   Error Percent

F= 200

Development:

Angle Measured = 60

Angle Calculated = 57.86

Ep = 57.86-6050(100) = 4.28              Ep = 4.3%

F= 300

Development:

Angle Measured = 44

Angle Calculated =46.7     Ep = 46.7-4446.7(100) = 5.78158         Ep = 5.8%

F= 400

Development:

Angle Measured = 36

Angle Calculated = 38.5

Ep = 36-38.538.5(100) = 6.49351         Ep = 6.5%

F= 500

Development:

Angle Measured = 36

Angle Calculated = 32.5

Ep = 36-32.532.5(100) = 10.7692          Ep = 10.8%

F= 600

Development:

Angle Measured = 28

Angle Calculated = 27.95

Ep = 27.95-2827.95(100) = .1789            Ep = .179%

F= 800

Development:

Angle Measured = 20

Angle Calculated = 21.7

Ep = 21.7-2021.7(100) = 7.8341			Ep = 7.8%

F= 1000

Development:

Angle Measured = 16

Angle Calculated = 17.65

Ep = 17.65-1617.65(100) = 9.34844		Ep = 9.35%

F= 1200

Development:

Angle Measured = 12

Angle Calculated = 14.9

Ep = 12-14.914.9(100) = 19.4631		Ep = 19.5%

F= 1500

Development:

Angle Measured = 12

Angle Calculated = 11.98

Ep = 12-11.9811.98(100) = .166945		Ep = .167%

Part B:

Circuit 1:

Voltage across 10Ω sensing resistor = 24mV

Current through the circuit = 1.9608mA

IT

2Vp-p

F=1khz

IT= VRT = 1V510Ω = 1.9608mA

V

500 Ω

10 Ω

Circuit 2:

Voltage across 10Ω sensing resistor = 19. 078mV

Current through the circuit =1.9608mA

10 Ω

IT

2Vp-p

F=1khz

IT= VRT = 1V510Ω = 1.9608mA

V

V10 = IT x R10 = 1.9608mA x 10 Ω = 19. 078mV

500 Ω

- The phase angle difference between two waveforms was measured in two different ways.  First, in the Time Interval Method I set both channels in the AC mode to the same vertical sensitivity setting.  Then, I adjusted the voltage to display both channels through the chop mode to measure them in terms interval.  Once I got it done, I obtained the distance from one waveform to another and its period as well.  Second, in the expanded waveform method I adjust the voltage and frequency along the time division.  Once both channels were displayed by the chop switch, I counted the tic marks between similar points and multiplied them by 4.

- The oscilloscope does not measure current.  However, you can obtain this through a resistor which I did.  I let the current flows through the resistor and connect the ends of this to the oscilloscope. I was able to follow on the screen the time variation of the intensity of the current, knowing the amplification of the oscilloscope to find I = V/R.

Comparing both phase angle measurement.  I conclude that the Angular Interval Method is more accurate than the Time Interval Method.  The reason why is because the error’s percent obtained from the Angular Interval Method is much smaller than the Time Interval Method’s error percent according with the calculated data.