A	     R1	B     R2    C      R3    D     R4       E

1k	             1k            2.7k        3.3k

RT

8K

IT

IT= 3/8k = .375mA

V	VR1= (1K) (.375mA) = +.375v

VR2= (1K) (.375mA) = +.375v

VR3= (2.7K) (.375mA) = +1.0125v

VR4= (3.3K) (.375mA) = +1.2375v

KVL @ VA

Ǿ= VDVM- VR4- VR3- VR2- VR1

Ǿ= VDVM- 1.2375v-1.0125v-.3750v-.3750v

Ǿ= VDVM-3v

VA = VDVM= +3v

KVL @ VB

Ǿ= VDVM- VR4- VR3- VR2

Ǿ= VDVM- 1.2375v-1.0125v-.3750v

Ǿ= VDVM-2.6350v

VA = VDVM= +2.6350v

KVL @ VC

Ǿ= VDVM- VR4- VR3

Ǿ= VDVM- 1.2375v-1.0125v

Ǿ= VDVM-1.250v

VA = VDVM= +1.250

KVL @ VD

Ǿ= VDVM- VR4

Ǿ= VDVM- 1.2375v

VD = VDVM= +1.2375v

VD = +1.2375v

KVL @ VE

VE= Ground

KVL @ VAB

VAB = VR1 = .3750v

VAB = +.3750v

KVL @ VAC

VAC = VR1 + VR2

VAC = .3750v + .3750v

VAC = +.750v

KVL @ VAD

VAD = VR1 + VR2 + VR3

VAD = .3750v + .3750v+ 1.0125v

VAD = +1.7625v

KVL @ VAE

VAE = VE = +3v

VAE = +3v

KVL @ VBC

VBC = VR2 = .3750v

VBC = +.3750v

KVL @ VBD

VBD = VR2 + VR3

VBD = + .3750v+ 1.0125v

VBD = +1.3875v

KVL @ VBE

VBE = VE + VR1

VBE = +3Vv - .3750v

VBE = +2.650v

KVL @ VCD

VCD  =  VR3

VCD  = + 1.0125v

KVL @ VCE

VCE  =  VR3+ VR4

VCE  =  +1.0125v +1.2375v

VCE  =  +2.250v

KVL @ VDE

VCE  =  VR4

VDE  =  +1.2375v

The information in this report was obtained through the calculated data along with the measurements made in the lab.  Errors obtained during the process of this experiment are result of differences between the resistors and the voltage applied in the circuit.  Moreover, the graphic data was obtained from the oscilloscope and sketched in order to know the waveforms reactions with the function of the DC and AC coupled mode.