Subject:

EET 111

Prof.:

Adam A. Fillios

Date:

Dic. 10, 2008

Made by:

Piter Garcia

Manual Lab Experiment # 8

Farmingdale Stage College

08

Circuit I

Piter Garcia

Experiment #8

Thevenin’s and Norton’ Equivalent Circuits

The purpose of this experiment is to determine the Thevenin and the Norton Equivalent Circuit of a given network both experimentally and theoretically.

IT

+                                                                                    3.3k 				  4.7k

Vin                                                                      A              B

2.7k				  6.8k

-

It is desired to obtain the Thevenin and the Norton Equivalent circuit “Looking” into terminals A-B.  When trying to calculate Rth looking into terminals you will find that series-parallel resistor combination techniques cannot be applied because there is no clear series-parallel combination.  In this experiment you will use measurement techniques and your knowledge of equivalent circuits to obtain Rth, Rn, Vth and then u se special calculations to verify our measurements.

- Set up the circuit.

- Assuming you can use only an Ohmmeter and a Voltmeter determine Rth, Rn, Eth, In. Show any additional calculations and show schematically how the available instruments should be connected.

- Repeat (2) with only an Ohmmeter and an Ammeter available.

- Repeat (2) with only a Voltmeter and an Ammeter available.

- Repeat (2) with only a power Supply and an Ammeter available.

- Use theoretical calculations only to determine Rth, Rn, Eth, and In.

Complete the Table

| Condition | Rth | Vth | In | Rn |
| --- | --- | --- | --- | --- |
| 2 | 4.160k | 5.130v | 1.233mA | 4.160k |
| 3 | 4.160k | 5.129v | 1.233mA | 4.160k |
| 4 | 4.161k | 5.130v | 1.233mA | 4.161k |
| 5 | 4.161k | 5.130v | 1.233mA | 4.161k |
| 6 | 4.178k | 5.003v | 1.197mA | 4.178k |

Calculation

The experiment developed above was resolved through Thevenin’s theorem.  This realization of this lab report experiment was made through steps; replacing the source voltages for a short circuit and the source currents for an open circuit.  In this way, we can deal so much easier with the circuit to find the Rthevenin’s. Once I got all the sources set in zero and replaced by short and open circuits,   I looked for the relation between resistors to realize which are parallel and which not.  Then I broke the circuit, numbering its bases A and B and setting the resistors shared by the previous points.  As soon as I broke the circuit and set it in a proper way, everything was much easier because I could identify the resistors in series and parallel without reducing the circuit in the typical way.  Also, the measurement made with the Ohmmeter, Ammeter, Voltmeter, and Power Supply helped to confirm the truly value of the Rthavenin.