Farmingdale  University of New York

08

Lab Manual Experiment

Electric Circuit I

Made by:

Piter Garcia

Subject:

EET 111

Prof:

Marjaneh Issapour

Date:

October, 16, 2008

Calculate and then measure the following voltages:

Va,  Vb,  Vab,  Vdb,  Vc,  Vda,  Vcd,  Vd

1k	   3.3k

Resistor Selected	IT		R1		     R2

R1=  1 K Ω	             a	               b	c

R2=  3.3 K Ω                                                                              +	- V1 +                     - V2 +          -	         R3

R3=  4.7 K Ω	      20V                                                                            V3           4.7

R4=  2.7 K Ω                                                                              -	+

RT=  11.7 K Ω                                                                                        2.7

-     V4   +

d

R4

IT = VT/RT = 20v/11.7 = 1.7mA

RT = R1 + R2 + R3 + R4 = 1k + 3.3k + 4.7k + 2.7k  	 RT = 11.7

VR1= (1k) (1.7k) = 1.7v					 VR4 = (2.7k) (1.7mA) = 4.59v

VR2= (3.3k) (1.7k) = 5.61v				 VR3 = (4.7K) (1.7K) = 7.99v

Vab= 20v (R1/RT) = 20v (1k/11.7k) = 1.709v	Va = VR4 + VR3 + VR2 + VR1

Vab= 1.709v 						Va = 4.59v + 7.99v + 5.6v + 1.7v

Va = 19.89v

Vcd = 4.7k + 2.7k = 7.4k

Vcd = 7.4k 							Vc = VR4 + VR3= 4.59v + 7.99v

Vc = 12.58v

Vb = VR4 + VR3+VR2 = 4.59v + 7.99v + 5.61v

Vb = 18.19v							Vda = Vd – Va = 4.59v – 19.89v

Vda = -15.3v		    Vdb = Vd – Vb = 4.59v – 18.19v = -13.6v

Measure Voltages:

Va = 19.913v

Vb= 18.119v

Vab = 1.7925v

Vdb = -13.515v

Vc = 12.035v

Vda = 15.309v

Vcd = 7.433v

Vd = 4.603v

Determine the value of R so that:

- Vxy = 8V, 		(b) Vxy = 4v       Then measure Vxy from each R.

2.7K                        1K

Resistor Selected	IT		R1		     R2

R1=  2.7 K Ω	             a	               x	y

R2=  1 K Ω                                                                              +	- V1 +                     - V2 +          -	         R3

R3=  	      16V                                                                            V3

RT=                                                                                             -	+

d

R4

Vxy = 8V

R3 = 8v + 3.7k = 16v						R3 = 3.7k

R3 = 8v + 1k + 2.7k = 16k	8v/3.7k = 3.7k

RT = R1 + R2 + R3 = 2.7K + 1K + 3.7K = 7.4K      IT = VT/RT = 16v/7.4k = 2.162mA

IT = 2.162mA

R3 = V/I = (4v/2.162mA) = 1.85k

R3= 1.85k

Measure Vxy = 8v and Vxy = 4v

- Vxy for a 3.5k = 9.7v

- Vxy for a 1.85k = 4.352v

Note:  (The board does not have a 3.7k)

Conclusion

The work developed above shows is a demonstration of how to find any value of a network only applying logic. The second circuit is an example of how to find values of its components without having specific values, but using common values between its components.  I applied some methods to find the first circuit’s values such as; Ohms Law, KVL, and Voltage Division. In the same way, I also applied Ohms Law and use logic to find the value of one of its resistor.  This posses the same voltage value than the others together, which makes it equal to the sum of the other resistors.  In fact, relationship between every single component of a network is to close, and all of them are related. If you have the value of two network’s components you may have the other value as far as you find where they relate one another.