The objective of this experiment is to measure the differential and common mode gains of a single op-am differential amplifier and calculate the rejection ratio to compare these values with those obtained from theory.

1.

Proto – Board, Model 203A

2.

Function Generator

3.

Oscilloscope

4.

Transformer

5.

M4741 Operational Amplifier

6.

Ohmmeter

7.

A 10k ohm resistor

8.

A 2k ohm resistor

9.

Various lengths of hook-up wire and instrument leads as required

Given Data:

Rf= Rb= 10k,	V+= 2vpp sine wave 1khz

Ri= Ra= 01k,	V- =1vdc

Gain= 5

Ri= ?

Ra= ?

Rf = ?

Part# 1:

Rb= ?

Solving for V-output

1)

Gain1=   RfRi = 10k02k = 5

Vout= (v+ - V-) RfRi =Vo = (1vp – -1vp) 10k02k Vo= 2* 5              Vo = 10

Output Voltage Predicted:

Part# 1:

Repeat by interchanging V+ and V-

Given Data:

Rf= Rb= 10k,	V+= 2vpp sine wave 1khz

Ri= Ra= 01k,	V- =1vdc

Gain= 5

Ri= ?

Ra= ?

Rf = ?

Rb= ?

Solving for V-output

2)

Gain1=   RfRi = 10k02k = 5

Vout= (V- - v+) RfRi =Vo = (-1vp – 1vp) 10k02k Vo= -2* 5              Vo = -10

Output Voltage Predicted:

Part# 3:

Rf = ?

Ra= ?

Rb= ?

Ri= ?

+

V in

cM

-

Given Data:

Rf= Rb= 10k,	V+ = 2vpp sine wave 1khz

Ri= Ra= 01k,	V- = 1vdc

Gain= 5

Solving for V-output

1)

Gain1=   RfRi = 10k02k = 5

Vout= (v+ - V-) RfRi =Vo = (1vp + -1vpk) 10k02k Vo= 0* 10k02k              Vo = 0

Theoretical Measure V+p V-p Voutp-p Voutp-p 1.03v -1v 10v 10.3v 1.03v -1v -10v -10.3v ACM CMdB Rf = ? Rb= ? Ra= ? Ri= ? 6.93m 57.16 + V in cM -

Rb= ?

Ra= ?

Ri= ?

Rf = ?

Rf = ?

Ri= ?

Ra= ?

Rb= ?

-

+

+

Measurements:

a) Max Vinp-p measured value

Vin= 18.6p-p

b) Vcmp-p output

VcM= 126.9mv

c) Common Mode Gain

ACM= VcMp-pVinp-p = 126.9mv18.6p-p = 6.93m

CMRR = ADACM = 56.934m = 720.93

CMRR = 720.93

CMRR dB= 20Log (720.93) =57.158

CMRR dB= 57.158

In the first problem, part one and two, the polarities of the ouputs were not affected by the gain but the polarity of their inputs. However, the magnitude of the output is increased and decreased according to the gain value. In the second problem, both voltage sources have the same values but different polarities, making the input and output voltages equal zero. Nevertheless, experimentally the output signal was affected by the noise given as an output a small signal.