Subject:

EET 111

Prof.:

Adam A. Fillios

Date:

October, 29, 2008

Piter Garcia

Farmingdale University of New York

Manual Lab Experiments #3 & 4

Electric Circuit I

Made by Piter Garcia

Analysis of Parallel Networks

IT                                      IA                                                IB	       IC	    I4

I1	                      I2                                           I3

16V                                                   1k	       2.7k                            3.3k	                           4.7k        			                     +

-

- Calculate all indicated currents in the above network, using the nominal values of the resistors.

Sources:     RT=                       1                     = 530.2      IT= VT/RT = 16/.5302 = 30.18mA

VT= 16v              1/1k + 1/2.7k + 1/3.3k + 1/4.7k              IT= 30.18mA

R1= 1k

R2= 2.7k           I2= V2/R2 = 16/2.7 = 5.926mA                IB= IA – I2 = 14.18mA -5.926mA

R3= 3.3k             I3= V3/R3 = 16/3.3k = 4.849mA              IB= 8.255mA

R4= 4.7k            I4= V4/R4 = 16/4.7 = 3.405mA            Ic= IB – I3 = 8.255mA - 4.849mA

I1= V1/R1 = 30.18mA/1k = 16mA        Ic=3.407mA

IA= IT - I1 = 30.18mA - 16mA

IA=14.18mA

- Measure all indicated currents and tabulate them along with calculated values in table 1.

|  | I4 | IC | I3 | IB | I2 | IA | I1 | IT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated | 3.405mA | 3.407mA | 4.849mA | 8.255mA | 5.926mA | 14.18mA | 16mA | 30.18mA |
| Measured | 3.37mA | 3.1mA | 4.65mA | 8.4mA | 5.76mA | 14.mA | 15.7mA | 29.54mA |

Analysis of Parallel Networks

IT

IA				           IB

+	I2		    I3	                 I2

Vin                                                   2.7k               3.3k	                       4.7k              6.8k

-

- Calculate the required Vin so that IT = 10mA.

- Set Vin to the voltage obtained in part (a) above.

- Using the current divider rule exclusively calculate IA, IB, I1, I2, I3, and I4.

(Using the nominal resistance values in your calculations.)

- Measure all the currents and tabulate them alon with their calclated values in Table 2.

Sources:                Rt=                       1                        = 967.8

Vin= 9.698v                      1/2.7k + 1/3.3k + 1/4.7k + 1/6.8k

R1 = 2.7k

R2 = 3.3k                Vin = Iin Rin = 10mA (969.8) = 9.698v

R3 = 4.7k

R4 = 6.8k

I1= IT (RT/R1) = 10(969.8/2.7k) = 3.592mA      IRA= IT (RT/RA) = 10mA (967.8/1.485k)

I2= IT (RT/R2) = 10(969.8/3.3k) = 2.939mA      IRA = 6.517mA

I3= IT (RT/R3) = 10(969.8/4.7k) = 2.063mA      IRB= IT (RT/RB) = 10mA (967.8/2.779k)

I4= IT (RT/R4) = 10(969.8/6.8k) = 1.426mA      IRB = 3.489mA

|  | IA | IB | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- | --- | --- |
| Calculated | 6.517mA | 3.489mA | 3.592mA | 2.939mA | 2.063mA | 1.426mA |
| Measured | 6.408mA | 3.503mA | 4.65mA | 2.84mA | 2.0mA | 1.3mA |

Experiment #4

Series Parallel Networks

Part 1: Theoretically solve each of the networks of figures 1 through 3 for the nodes voltages VA and VB (i.e. the voltage from node A to ground and the voltage from node B to ground.) Record your calculated values in the tables, and answer the following questions:

Compare the values of VA and VB in the three networks. How are they related?

Even though the networks have different magnitudes, their relationship between nodes VA and VB are the same. They are dependent; it means that they depend on other sources to produce their own.  In this case, VA and VB are depending on the node E, which is the reason why both, their currents and voltages may change in magnitude or polarity according with the independent node(s) in the network.

What “principle” does this demonstrate?

This demonstrates that the magnitude and polarity of VA and VB will always depend on the node E.  In other words, node E will always be an independent node, which provides the total current and voltage to a network.

Part 2: Set up each of the networks of figures 1 through 3 in turn, and experimentally measure the node voltages (E, VA, and VB) of each network.

Record your reading for each network in the corresponding tables.

| Fig. 1 | Calculated | Measured |
| --- | --- | --- |
| E | 10 | 10.065v |
| VA | 4.512v | 4.5v |
| VB | 1.887v | 1.9v |

| Fig. 2 | Calculated | Measured |
| --- | --- | --- |
| E | 20 | 20v |
| VA | 9.024 | 8.9v |
| VB | 3.772 | 3.7v |

| Fig. 2 | Calculated | Measured |
| --- | --- | --- |
| E | -20 | -20v |
| VA | -9.024 | -8.9v |
| VB | -3.772 | -3.7v |

Conclusion

The work developed above was made by reducing the circuit in a simple series circuit.  We can notice that both, the first and second experiments of parallel networks can be solved by many ways or formulas.  However, the reason why I decided to solve it by reducing in a simple series circuit is because it allows me not just to get the current through every resistor, but also the current shared by two or more resistors.  In this way, currents such as IA, IB, and IC shown in the networks of the experiments above, are so much easier to find with an exacter result. Furthermore, the measuring of the networks in the lab confirmed the credibility of the results and process applied during in these experiments.  In fact, these experiments helped me to apply and develop my knowledge of the relationship between dependent and independent nodes and their functions in a network.