Lab Manual Experiment #2

Electric Circuit I

Made by:

Piter Garcia

Subject:

EET 111

Prof:

Marjaneh Issapour

Date:

10/08/08

2008

Farmingdale  University of New York

-

29-9-2008

Experiment #2

Analysis of Series Networks

The purpose of this experiment is to illustrate the principles governing a series network.

Procedures:

- Set up a seires circuit with four resistors from the board so that approximately 2mA of current are drawn from a 20V supply.  Show your calculations for the resistor selection.

Resistor Selected	IT		R1		R2

R1=  3.3 K Ω

R2=  4.7 K Ω	+V1 -                     + V2 -          +	         R3

R3=  1 K Ω	      20V                                                                            V3

R4=  1 K Ω	-       	R4

RT=  10 K Ω                                                                                                                                    +

V4

-

- Calculate the values of RT= R1 + R2 + R3 + R4	I= 20/RT

RT =  R1 + R2 + R3 +R4 = 3.3K Ω + 4.7K Ω + 1K Ω + 1K Ω = 10K Ω

I = 20v/10K Ω =  2mA

- Measure and tabulate measured and calculated values of the following quantities.

|  | V1 | V2 | V3 | V4 | I |
| --- | --- | --- | --- | --- | --- |
| Calculated | 6.6 | 9.4 | 2v | 2v | 2mA |
| Measured (VOM) | 6.9 | 8.9 | 2.2v | 2.1v | 1.92mA |

- Verify that KVL is satisfied using your measured values.

Calculated:

Ǿ =  VT – V1 – V2 – V3 – V4 	  	 20V – 6.6V – 9.4V – 2V – 2V

=  20V – 16V – 4V =	20V – 20V	=   0

Ǿ = Ǿ

Mesured:

Ǿ =  VT – V1 – V2 – V3 – V4	      	 20V – 6.8V – 8.9V – 2.2V – 2.1V

=  20V – 15.7V – 4.3V = 20V – 20V	=   0

Ǿ = Ǿ

- Measure the voltage between points A and B and use the measured value to predict V3 and V4 using the volate divider rule.  Compare the result to the values obtained in part(3).

V3=  VT ( R3/RT ) = 20V(1 K Ω /10 K Ω) = 2V				V3= 2V

V4=  VT ( R3/RT ) = 20V(1 K Ω /10 K Ω) = 2V				V4= 2V

- The voltage A-B is to remain fixed to the value obtained in part (5) while the input voltage is increased to 30 volts.    Verify that indeed VAB has not changed.

V1 + V2 + V3 + V4 = 30V			V1 + 9.4V + 2V + 2V = 30V

V1 + 13.4V = 30V 				V1 = 30V - 13.4V

V1  = 16.6V

Predict the value of R1, required to accomplish this and then replace R1 by a resistor box set to your predicted value.

R1 = V1/IT   =   16.6v/2mA  =  8.2V		R1= 8.2V

Conclusion

Throughout this process I had the opportunity to practice what I have been learning in class.  I have learned how to predict almost the axactly value of a network’s components such as resistor, volt, and current without calculating but using a Digital Multimiter.  Furthermore, I could see, touch, and work with a circuit siries developing this experiment.  I alson analysed it manually, knowing personally every single component of this network.

Check out circuits A1 to A10 (make sure the A/B switch is in the A position).  For each part, determine:

- The minimun number of switch throws required.

- The boolean equation.

- 2)

A1			             (A1)		          A2				  (A2)

| A | B | A1 |
| --- | --- | --- |
| O | O | O |
| O | 1 | O |
| 1 | O | O |
| 1 | 1 | 1 |

| A | B | A2 |
| --- | --- | --- |
| O | O | O |
| O | 1 | 1 |
| 1 | O | 1 |
| 1 | 1 | 1 |

A		B	       A1=  (A . B)			A	         B		            A2= (A + B)

(A4)

| A | B | C | A4 |
| --- | --- | --- | --- |
| O | O | O | O |
| O | O | 1 | 1 |
| O | 1 | O | 1 |
| O | 1 | 1 | 1 |
| 1 | O | O | 1 |
| 1 | O | 1 | 1 |
| 1 | 1 | O | 1 |
| 1 | 1 | 1 | 1 |

A3              		             (A3)		         A4

| A | B | A3 |
| --- | --- | --- |
| O | 1 | O |
| O | O | O |
| 1 | 1 | 1 |
| 1 | O | O |

A		B	        A3= (A . B)		         A	     B	C		     A4=  [(A + B) C]

| A | B | C | D | A5 |
| --- | --- | --- | --- | --- |
| O | O | O | O | O |
| O | O | O | 1 | 1 |
| O | O | 1 | O | 1 |
| O | O | 1 | 1 | 1 |
| 1 | 1 | O | O | 1 |
| 1 | 1 | O | 1 | 1 |
| 1 | 1 | 1 | O | 1 |
| 1 | 1 | 1 | 1 | 1 |

A5)  					                        	            (A4)

T

P            E

A5=  [A (B . C) D]

A       B       C         D	A5=  (P . E . T)

(A6)

T

P

R

G

A6= (P . T)

A       B        C         D         E	A6=  [(A + B) (C + D) E]

(A7)

P	  T

E

A8= [(A + B) (C . D . D) E]

A7=  (A + B) (B . C) (D . E)

A             B              C              D   E	                              A7= (P . E . T)

(A8)

P	        T

E

A8= (P. E . T)

A B C D E

(A9)

T

P	E

A9= (A + B) (C + D) (D . E)

A	            B             C        D	    E	A9= (P. E . T)

(A10)

P             	T

E

A10= ( P . E . T)

A       B       C         D         E          F

A10=  [(A . B) C] (D + E) (E + F)