| Low Pass Filter |  |  |  |  |
| --- | --- | --- | --- | --- |
| Hz Frequency | Vin-peak | Vout-peak | Phase | AdB |
| 100 | 8v | 7.3v | 7° | -.796 |
| 200 | 8v | 7.2v | 13° | -.915 |
| 500 | 8v | 6.4 | 29° | -1.938 |
| 1k | 8v | 5v | 43° | -4.083 |
| 2k | 8v | 3.1v | 60° | -8.235 |
| 5k | 8v | 1.4v | 70° | -15.139 |
| 10k | 8v | 730mv | 84° | -20.795 |
| 20k | 8v | 380mv | 85° | -26.466 |
| 50k | 8v | 1.36mv | 88° | -75.341 |
| 100k | 8v | 67mv | 86° | -41.54 |
| 200k | 8v | 31mv | 90° | -48.235 |
| 500k | 8v | 11.9mv | 80° | -56.551 |

| High Pass Filter |  |  |  |  |
| --- | --- | --- | --- | --- |
| Hz Frequency | Vin-peak | Vout-peak | Phase | AdB |
| 100 | 8v | 800mv | -83° | -20 |
| 200 | 8v | 1.50v | -80° | -14.54 |
| 500 | 8v | 3.6v | -63° | -6.936 |
| 1k | 8v | 5.5v | -43° | -3.255 |
| 2k | 8v | 6.7v | -20° | -1.54 |
| 5k | 8v | 7.5v | -11° | -0.56 |
| 10k | 8v | 7.5v | -5° | -0.56 |
| 20k | 8v | 7.5v | -2° | -0.56 |
| 50k | 8v | 7.5v | 0° | -0.56 |
| 100k | 8v | 7.5v | 0° | -0.56 |
| 200k | 8v | 7.5v | 0° | -0.56 |
| 500k | 8v | 7.5v | 0° | -0.56 |

Summing up the data obtained from the high and low frequency plots I conclude the following. First, the low pass signal drops 3dB voltage in gain dB from 100Hz to 1 kHz. On the other hand, the high pass signal increases 3dB voltage in gain dB. At 50 kHz both signals are facing one another with about 90 angle degree. You can observe that at 50Hz the High pass signal V-output equals the V-input, and the low pass signal reaches its lowest voltage in gain dB.

fB= 12πRC = 12π68(2400pf) = 975.214 Hz

High Pass Filter`

VoutVin = 11-j fbf = 11-j 975.214100 = 102.007m ∠ 84.135°

Vout =  Vin * A = 8 *102.007m = 816.054mv

AdB = 20 log(102.007m) = -19.827 ∠ 84.135°

VoutVin = 11-j fbf = 11-j 975.2141K = 715.924m∠ 44.281°

Vout =  Vin * A = 8 *715.924m = 5.727

AdB = 20 log(715.924m)= -2.903 ∠ 44.281°

VoutVin = 11-j fbf = 11-j 975.21410K = 995.278m ∠ 5.57°

Vout =  Vin * A = 8 *995.278m =7.962

AdB = 20 log(995.278m)=-.041∠ 5.57°

VoutVin = 11-j fbf = 11-j 975.214100K = 999.952m∠ 0.558°

Vout =  Vin * A = 8 * 999.952m = 7.996

AdB = 20 log (999.952m)= -.0004∠ 0.558°

VoutVin = 11-j fbf = 11-j 975.21450K = 999.81m∠ 1.12°

Vout =  Vin * A = 8 * 999.81m = 7.999

AdB = 20 log (999.981m)= -1.65∠ 1.12°

Low Pass Filter

VoutVin = 11-j fbf = 11+j 100975.214 = 994.214m ∠ 5.85°

Vout =  Vin * A = 8 * 994.214m = 7.958v

AdB = 20 log  (994.214m)= 50.4m∠ 5.9°

VoutVin = 11-j fbf = 11+j 1k975.214 = 698.179m∠ 45.719°

Vout = Vin * A = 8 * 698.179m = 5.585v

AdB = 20 log (698.179m)= -3.121∠ 45.719°

VoutVin = 11-j fbf = 11+j 10k975.214 = 97.061m∠ 84.43°

Vout =  Vin * A = 8 * 97.061m = 776.488mV

AdB = 20 log (97.061m)= -20.259∠ 84.43°

VoutVin = 11-j fbf = 11+j 100k975.214 = 9.752m∠ 89.441°

Vout =  Vin * A = 8 * 9.752m = 78.013mV

AdB = 20 log (9.752m)= -40.218∠ 89.441°

VoutVin = 11-j fbf = 11+j 50k975.214 = 19.5m∠ 88.9°

Vout =  Vin * A = 8 * 19.5m = 156mV

AdB = 20 log (19.5m)= -34.2∠ 88.9°