The objective of this experiment is to investigate the summing circuits of six op-amps along with its experimental result to compare with the theoretical result to make assuming an ideal op-amp.

1.

Proto – Board, Model 203A

2.

Function Generator

3.

Oscilloscope

4.

Transformer

5.

M4741 Operational Amplifier

6.

Ohmmeter

7.

A 2k ohm resistor

8.

A 4k ohm resistor

9.

A 6k ohm resistor

10.

Various lengths of hook-up wire and instrument leads as required

V1= 1Vp-p

V2= 1Vdc

A= 1 + RfRi = 1+100k10k = 11

Super-Position:

V1) V-out= (A) (Ri || Ra)(Ra+ (Ri || Ra)) (Vin)

V-out= (11) (10k || 10k)(5k+ (10k || 10k)) (1vp-p) = (11) 5k10k (1) = 5.5v

V-out= 5.5v

V2) V-out= (A) (Ri || Ra)(Ra+ (Ri || Ra)) (Vin)

V-out= (11) (5k || 10k)(10k+ (5k || 10k)) (1vp-p) = (11) 3.33k10k+3.33k (1) = 2.75v

Vout1,2= 5.5v + 2.75v = 8.25

V= 8.25

| All Measurements |  |  |
| --- | --- | --- |
|  | Theoretical | Experimental |
| V1 | 5.5vp-p | 5.31p-p |
| V2 | 2.75v | 2.78v |
| VT | 8.25v | 8.1v |

As we can observe in the experiment above, the Inaccuracies in the resistor values will lead to inaccuracies in the calculated output voltage. Throughout the experiment we proved that the accuracy of the output depends on the accuracy of the inputs.