The objective of this experiment is to investigate the summing circuits of six op-amps along with its experimental result to compare with the theoretical result to make assuming an ideal op-amp.

1.

Proto – Board, Model 203A

2.

Function Generator

3.

Oscilloscope

4.

Transformer

5.

M4741 Operational Amplifier

6.

Ohmmeter

7.

A 2k ohm resistor

8.

A 4k ohm resistor

9.

A 6k ohm resistor

10.

Various lengths of hook-up wire and instrument leads as required

V1= 1Vp-p

V2= 1Vdc

A= 1 + RfRi = 1+100k10k = 11

Super-Position:

V1) V-out= (A) (Ri || Ra)(Ra+ (Ri || Ra)) (Vin)

V-out= (11) (10k || 10k)(5k+ (10k || 10k)) (1vp-p) = (11) 5k10k (1) = 5.5v

V-out= 5.5v

V2) V-out= (A) (Ri || Ra)(Ra+ (Ri || Ra)) (Vin)

V-out= (11) (5k || 10k)(10k+ (5k || 10k)) (1vp-p) = (11) 3.33k10k+3.33k (1) = 2.75v

Vout1,2= 5.5v + 2.75v = 8.25

V= 8.25

| All Measurements |  |  |
| --- | --- | --- |
|  | Theoretical | Experimental |
| V1 | 5.5vp-p | 5.31p-p |
| V2 | 2.75v | 2.78v |
| VT | 8.25v | 8.1v |