The purpose of this experiment is to illustrate that the principle of superposition can be applied to circuits with dependent and independent sources.

4)

D) Verify the principle of superposition, by adding the individual responses (obtained in parts b and c) and comparing them with the total responses (obtained in part a)

|  | V1 | V2 | V3 | V4 | V5 |
| --- | --- | --- | --- | --- | --- |
| B | 58 | 55 | 0 | 65 | -15 |
| C | 20 | 20 | 20 | 20 | 0 |
| B+C | 78 | 75 | 20 | 85 | -15 |
| A | 78 | 75 | 20 | 85 | -15 |

5)

| 5a | V1 | V2 | V3 | V4 | V5 |
| --- | --- | --- | --- | --- | --- |
|  | 58 | 55 | 0 | 65 | -15 |

I1= 1mA		V3=0

VR1= R1*I1= 3k*1mA= 3v

VR1= 3v

VR3= R3*10*I1= 1k*10mA

VR3= 10v

V1= VR1 + VR2= 3v + 55v= 58v

V1= 58v

V2= VR2= 55v

V2=55v

V3=0

V1, 2= VR1= 3v

V2, 3= VR2= 55v

VR2= R2*(I1+ 10*I1)

VR2= 5K*(1mA+10mA)= 55v

VR2= 55v

VR4= R4*10*I1= 1.5K*10mA

VR4= 15V

V4= VR3+VR2= 10v + 55v= 65v

V4= 65v

V5= -VR4= -15v

V5= -15v

|  | V1 | V2 | V3 | V4 | V5 |
| --- | --- | --- | --- | --- | --- |
| 5b | 20 | 20 | 20 | 20 | 0 |

I1= 0		V3= 20

VR1= R1*I1= 3k*0mA= 0v

VR1= 0v

VR3= R3*10*I1= 1k*0mA

VR3= 0v

V4= VR3+VR2 -20 =

V4= 0v + 0v -20 = 20v

V4= 20v

V5= -VR4= -0v

V5= 0V

VR2= R2*(I1+ 10*I1)

VR2= 5K*(0mA+0mA)= 0v

VR2= 0v

VR4= R4*10*I1= 1.5K*0mA

VR4= 0v

V1= VR1 + VR2= 0v + 0v -20

V1= 20v

V2= VR2 - 20= 0 - 20

V2=20v

V3= 20

V1, 2= VR1= 0v

V2, 3= VR2= 0v

|  | V1 | V2 | V3 | V4 | V5 |
| --- | --- | --- | --- | --- | --- |
| 5a | 58 | 55 | 0 | 65 | -15 |
| 5b | 20 | 20 | 20 | 20 | 0 |
| 5a+b | 78 | 75 | 20 | 85 | -15 |

| Comparison Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | V1 | V2 | V3 | V4 | V5 |
| Theoretical | 78 | 75 | 20 | 85 | -15 |
| Experimenta | 78 | 75 | 20 | 85 | -15 |
| % Error | 0 | 0 | 0 | 0 | 0 |